# VU13P — design notes

Project: vu13p.xpr · Vivado 2025.2 · part xcvu13p-fhgb2104-2-i · top aurora_gty_vu13p_rst_wrapper Source: copied from ~/Desktop/PJ22045_EV1/13P_integrated_2 (original untouched). GT pinout reference (read-only): ~/Desktop/board_test_item/test_item_13p/project/ibert_13pTo47DR1_ex, ibert_13pTo47DR2_ex.

目錄

v1.0.0_Debug7 修改（Rex，2026-10-17）

系統中的角色（雙向）

Quad 用途（更新）

dbg_hub 時脈（constrs_1/new/target.xdc）

Build 結果（reports/）

待確認

Role in the system (1R8T)

Aurora 64B66B cores (all 2-lane, 25.625 Gbps, 156.25 MHz MGTREFCLK0 of own quad)

Refclk pins (constrs_1/new/physical.xdc, from golden IBERT)

Clocks / resets (unchanged from original)

Scripts (in ../scripts, run with vivado -mode batch -source ... from this dir)

Naming convention (2026-10-14, scripts/naming/)

Open items / assumptions

History

（若目錄沒有顯示：在 Word 中按右鍵 → 更新功能變數）

# v1.0.0_Debug7 修改（Rex，2026-10-17）

本節是 v1.0.0_Debug7 新增的內容（中文）。從「Role in the system」開始是同事的英文原文，沒有修改，描述的是 v1.0.0 的設計。

專案：source_code_v1.0.0_Debug7/VU13P/vu13p.xpr；修改腳本 scripts/debug007_bidir.tcl；build 腳本 scripts/build_vu13p.tcl。

## 系統中的角色（雙向）

正常：z1 (47dr1) ADC ch00 ──X1Y8 C1──► axis_slr_x1y8_to_tx ──► axis_broadcaster_0 (1→8) ──► X1Y4~X1Y7 ──► z2 → 8 DACs

交換：z2 (47dr2) ADC ch00 ──X1Y4 C1──► axis_slr_x1y4_to_z1 ──► axis_broadcaster_1 (1→8) ──► X1Y8~X1Y11 ──► z1 → 8 DACs   ← 新增

- 兩組 broadcaster 都是 HAS_TREADY = 0，不需要方向選擇：RFSoC_RX 只驅動 S_AXIS_X0Y1_C1，RFSoC_TX 不驅動任何 S_AXIS。所以放 RFSoC_RX 的那一側有資料，另一組 broadcaster 沒有輸入，保持閒置。

- 新路徑的設定、時脈（clk_wiz_0/clk_out1 400 MHz）和 reset（util_vector_logic_0/Res）都跟原路徑相同。Container aurora_gty_vu13pto47dr.bd 沒有修改，只是接上原本沒接的 M_AXIS_X1Y4_C1 和 S_AXIS_X1Y8~11_C0/C1。

| 新增 cell | 設定 |
| --- | --- |
| axis_slr_x1y4_to_z1 | axis_register_slice，REG_CONFIG 15（SLR crossing），NUM_SLR_CROSSINGS 1，PIPELINES_MASTER / SLAVE 1 |
| axis_broadcaster_1 | NUM_MI 8，HAS_TREADY 0，16 byte tdata，TKEEP / TLAST |
| ila_bcast_1 | system_ila，9 個 AXIS slot、1024 深；slot 0 = 從 z2 收到的資料，slot 1~8 = M00~M07（送 z1） |

## Quad 用途（更新）

| Quad | Bank | 對端 | v1.0.0 | v1.0.0_Debug7 新增 |
| --- | --- | --- | --- | --- |
| X1Y4 | 224 | z2 X0Y1 | TX：broadcaster_0 M00 / M01 | RX：core1 M_AXIS_X1Y4_C1 → axis_broadcaster_1 |
| X1Y5~X1Y7 | 225~227 | z2 X0Y2~4 | TX：broadcaster_0 M02~M07 | — |
| X1Y8 | 228 | z1 X0Y1 | RX：core1 → axis_broadcaster_0 | TX：broadcaster_1 M00 / M01 → S_AXIS_X1Y8_C0 / C1 |
| X1Y9 | 229 | z1 X0Y2 | 只有 link | TX：M02 / M03 |
| X1Y10 | 230 | z1 X0Y3 | 只有 link | TX：M04 / M05 |
| X1Y11 | 231 | z1 X0Y4 | 只有 link | TX：M06 / M07 |

## dbg_hub 時脈（constrs_1/new/target.xdc）

只加 ILA 的版本 timing 失敗（WNS −0.188 ns，83 個 endpoint），失敗的 path 全部是 dbg_hub 的 XSDB 路徑。原因是 dbg_hub 掛在 Aurora x1y9 的 user_clk 上（[get_nets clk]，約 400 MHz），多一個 ILA 之後 XSDB 路徑要跨 3 個 SLR。修改後：

set_property C_CLK_INPUT_FREQ_HZ 100000000 [get_debug_cores dbg_hub]                               # 原 300000000

connect_debug_port dbg_hub/clk [get_nets {aurora_gty_vu13p_rst_i/util_ds_buf_0_IBUF_OUT[0]}]   # 原 [get_nets clk]

原設定保留成註解，原檔是 scripts/target.xdc.orig。另一個好處：sys_clk 一直在跑，Aurora link 沒起來時也可以連 ILA / VIO。

## Build 結果（reports/）

| 項目 | v1.0.0（rev0.12 build #6） | v1.0.0_Debug7 |
| --- | --- | --- |
| WNS / WHS | +0.011 / +0.010 ns | +0.006 / +0.010 ns（最差：axis_slr_x1y4_to_z1 → ila_bcast_1 probe） |
| LUT / FF / BRAM | 3.26% / 4.69% / 18.36% | 3.38% / 4.66% / 17.97% |
| Power | — | 30.872 W |
| GT / refclk 位置 | golden | 與 v1.0.0 完全相同 |

## 待確認

- 尚未上板。X1Y4 core1 的 RX 方向跟 v1.0.0 已經在用的 VU13P → z2 方向用同一對 lane（Aurora 是 duplex，channel_up 需要雙向都通），但 z2 送 ADC 資料這件事還沒實測。

- pma_rst 輸入接到哪一顆 RFSoC 還不知道（兩顆的 PS 對 PL_RESET 的動作相同）。

# Role in the system (1R8T)

z1 (47dr1) ADC ch00 ──2-lane──► VU13P X1Y8 core1 ──► axis_broadcaster_0 (1→8) ──► 8× 2-lane ──► z2 (47dr2) → 8 DACs

- RX from z1: 8 cores on quads 228–231 (mirror of z1's 8 cores) so every z1 lane pair has a partner.

Only X1Y8 core1 (lanes X1Y34-35) carries data (z1 ADC ch00, from z1 X0Y1 core1 / X0Y6-7):

link_x1y8 → M_AXIS_X1Y8_C1 → axis_slr_x1y8_to_tx  (core1 again since 2026-10-14) (axis_register_slice, Multi_SLR_Crossing, 1 crossing,

+1 pipeline each side) → axis_broadcaster_0 — quad 228 and quads 224–227 are in different SLRs.

- TX to z2: 8 cores on quads 224–227, each fed by one broadcaster output.

- 13P↔13P link (aurora_gty_vu13ptovu13p, Quad_X0Y11 / X0Y13) kept unchanged; its RX is no longer the broadcaster source.

# Aurora 64B66B cores (all 2-lane, 25.625 Gbps, 156.25 MHz MGTREFCLK0 of own quad)

Container BD: aurora_gty_vu13pto47dr.bd (instance aurora_gty_vu13pto47_0 in top BD aurora_gty_vu13p_rst.bd). Every quad N = 4..11 has the same two blocks at container top level:

- hier_x1yN — 2 Aurora cores (core0 owns the QPLL/user clock, core1 shares it) + status VIOs + reset helpers.

- link_x1yN — identical copy of the original X1Y4 plumbing:

  - TX: S_AXIS_X1YN_C0 → tx_fifo_c0 (axis_aclk → user_clk) → tx_slice_c0 → core0;

S_AXIS_X1YN_C1 → tx_fifo_c1 → tx_slice_c1 → core1

  - RX: core0 → rx_slice_c0 → rx_fifo_c0 (user_clk → axis_aclk) → M_AXIS_X1YN_C0;

core1 → rx_slice_c1 → rx_fifo_c1 → M_AXIS_X1YN_C1

  - resets: user_rst_sync_c0/c1 (core channel-up rstn → user_clk), axis_rst_sync_c0/c1 (→ axis_aclk)

- system_ila_0 (debug) monitors X1Y4 only: core0 RX, core0 TX, core1 TX at the hier_x1y4 boundary.

| Quad | Bank | core0 lanes | core1 lanes | Peer | Use |
| --- | --- | --- | --- | --- | --- |
| X1Y4 | 224 | X1Y16-17 | X1Y18-19 | z2 X0Y1 | TX: broadcaster M00 → S_AXIS_X1Y4_C0, M01 → S_AXIS_X1Y4_C1 |
| X1Y5 | 225 | X1Y20-21 | X1Y22-23 | z2 X0Y2 | TX: M02 / M03 |
| X1Y6 | 226 | X1Y24-25 | X1Y26-27 | z2 X0Y3 | TX: M04 / M05 |
| X1Y7 | 227 | X1Y28-29 | X1Y30-31 | z2 X0Y4 | TX: M06 / M07 |
| X1Y8 | 228 | X1Y32-33 | X1Y34-35 | z1 X0Y1 | RX: core1 M_AXIS_X1Y8_C1 → axis_broadcaster_0 |
| X1Y9 | 229 | X1Y36-37 | X1Y38-39 | z1 X0Y2 | link only |
| X1Y10 | 230 | X1Y40-41 | X1Y42-43 | z1 X0Y3 | link only |
| X1Y11 | 231 | X1Y44-45 | X1Y46-47 | z1 X0Y4 | link only |

Unused container AXIS ports (TX inputs of X1Y8–X1Y11, RX outputs other than M_AXIS_X1Y8_C1) are left open (inputs tied to 0 by IPI).

# Refclk pins (constrs_1/new/physical.xdc, from golden IBERT)

| Port | Quad | P / N | Golden |
| --- | --- | --- | --- |
| gt_x1y4_diff_ref | 224 | AW9 / AW8 | 13pTo47DR2 q0 |
| gt_x1y5_diff_ref | 225 | AT11 / AT10 | 13pTo47DR2 q1 |
| gt_x1y6_diff_ref | 226 | AM11 / AM10 | 13pTo47DR2 q2 |
| gt_x1y7_diff_ref | 227 | AH11 / AH10 | 13pTo47DR2 q3 |
| gt_x1y8_diff_ref | 228 | AD11 / AD10 | 13pTo47DR1 q0 |
| gt_x1y9_diff_ref | 229 | Y11 / Y10 | 13pTo47DR1 q1 |
| gt_x1y10_diff_ref | 230 | T11 / T10 | 13pTo47DR1 q2 |
| gt_x1y11_diff_ref | 231 | M11 / M10 | 13pTo47DR1 q3 |

All 6.4 ns create_clock; each aurora_x1yN TX out clock is async to sys_clk. GT serial pins are placed by the Aurora IP location (no PACKAGE_PIN needed). pblock_1 (X1Y4 GT region) kept from the original.

# Clocks / resets (unchanged from original)

- sys_clk 100 MHz (BA15) → init_clk; clk_wiz_0 400 MHz → s_axis_aclk (broadcaster / FIFOs).

- pma_rst (AU16/AV16 LVDS) → hier_rst/ini_system → pma_init of all cores.

# Scripts (in ../scripts, run with vivado -mode batch -source ... from this dir)

- mod_vu13p.tcl — pass 1: x1y8 RX-only, 3 TX quad copies (X1Y5-7), broadcaster 1→8.

- mod2_vu13p_a.tcl — pass 2a: hier_x1y8 rebuilt with 2 cores + hier_x1y9..x1y11 (mirror of z1).

- mod2_vu13p_b.tcl — pass 2b: top-level ports, wrapper, top.

- mod5_vu13p.tcl — broadcaster source x1y8 core1 (superseded by pass 7).

- mod7_vu13p.tcl / mod7b_vu13p.tcl / mod7c_vu13p.tcl — uniform link_x1yN for all quads, broadcaster ←

M_AXIS_X1Y8_C1, ILA re-pointed.

- mod9_vu13p.tcl — SLR-crossing register slice in front of the broadcaster.

- run_build.sh vu13p [jobs] → build.tcl — clears stale container cache, synth → impl → bitstream, reports in reports/.

Gotcha: a BD container parent does not refresh when the child only *adds* ports — delete vu13p.gen/sources_1/bd/aurora_gty_vu13p_rst/bd/aurora_gty_vu13pto47dr_inst_0 before reopening the top BD.

# Naming convention (2026-10-14, scripts/naming/)

Every name is derived from what it is connected to (quad, Aurora core, direction); generated and checked by scripts/naming/gen_container.py / gen_top.py, applied with apply_container.tcl / apply_top.tcl (lib.tcl).

| Level | Name |
| --- | --- |
| Container / top GT ports | GT_SERIAL_TX_<QUAD>_C<n>, GT_SERIAL_RX_<QUAD>_C<n> (n = Aurora core 0/1 of that quad) |
| Container data ports | S_AXIS_<QUAD>_C<n> → TX of core n, M_AXIS_<QUAD>_C<n> ← RX of core n |
| Refclk / board ports | unchanged (gt_<quad>_diff_ref, XDC) |
| hier_<quad> pins | GT_SERIAL_TX_C<n>, GT_SERIAL_RX_C<n>, S_AXIS_TX_C<n>, M_AXIS_RX_C<n>, GT_REFCLK, arstn_c<n>, pma_init, user_clk_out |
| TX/RX/link hierarchy pins | S_AXIS_TX_C<n>, M_AXIS_TX_C<n>, S_AXIS_RX_C<n>, M_AXIS_RX_C<n>, user_clk (Aurora), axis_aclk (fabric), arstn_c<n> |
| Per-core sub-hierarchies / leaf cells | tx_c<n>, rx_c<n>; tx_fifo_c<n>, tx_slice_c<n>, rx_slice_c<n>, rx_fifo_c<n>, user_rst_sync_c<n>, axis_rst_sync_c<n>, cdc_fifo, in_fifo, out_fifo |
| Wires (all levels) | named after their driver: <cell>_<pin>, or the input port / hierarchy input pin driving it |
| Kept for XDC / software | Aurora core cells aurora_<quad>_<n>, hier_<quad>, container instance names, AXI peripherals (address map) |

# Open items / assumptions

- Lane-level wiring 13P↔47DR assumed straight (X1Y32-33 ↔ z1 X0Y4-5, etc.); golden IBERT only confirms quad pairing.

- Unused cores' TX AXIS inputs are left unconnected (tvalid = 0); RX outputs of x1y4..x1y7 / x1y9..x1y11 unused.

- Build #3 bitstream predates the 2026-10-14 naming pass; rebuild needed.

# History

- 2026-10-13 Project created from 13P_integrated_2 (srcs + xpr only); project_1.gen refs renamed to vu13p.gen.

- 2026-10-13 Pass 1 (mod_vu13p.tcl): removed x1y8 core1 + its TX/RX plumbing; copied hier_x1y4 TX chain to X1Y5/6/7

(tx_q5..7); broadcaster_0 NUM_MI 2→8 fed from M_AXIS_x1y8; removed broadcaster_1; XDC refclk 225–227 added.

BD validated, wrapper regenerated.

- 2026-10-13 Another Claude session re-ran mod_vu13p.tcl (21:04) and generated into a stray project_1.gen;

BD structure verified intact, stray dir removed, refs repointed to vu13p.gen.

- 2026-10-13 Pass 2 (mod2_vu13p_a/b.tcl): per user, z1 now uses 8 cores → hier_x1y8 rebuilt with 2 cores,

added hier_x1y9/x1y10/x1y11 (quads 229–231); XDC refclk 229–231 + clock groups. Broadcaster source unchanged

(x1y8 core0). BD validated, wrapper regenerated, top = aurora_gty_vu13p_rst_wrapper.

- 2026-10-13 Per user (option b): broadcaster source moved from x1y8 core0 to x1y8 core1 to match z1, where ADC1

is on X0Y1 core1 (S_AXIS_X0Y1). axis_register_slice_6 now takes hier_x1y8/USER_DATA_M_AXIS_RX1; its reset and

axis_data_fifo_7's reset moved from arstn to arstn1 (mod5_vu13p.tcl, top refreshed with mod2_vu13p_b.tcl).

BD validated, wrapper regenerated.

- 2026-10-13 Per user: all links made identical to X1Y4 (mod7_vu13p.tcl): X1Y4's FIFO/register-slice/CDC plumbing

grouped as link_x1y4 and copied to link_x1y5..link_x1y11 (old tx_q5–7 groups and x1y8 slice/FIFO removed);

new container ports S_AXIS_x1y8..11[_1], M_AXIS_x1y5..11[_1]. Broadcaster now fed from M_AXIS_x1y8_1

(mod7b_vu13p.tcl). Grouping created monitor-only boundary pins that caused ILA width-mismatch CRITICAL

WARNINGs → removed from all links and system_ila_0 re-pointed to the hier_x1y4 core interfaces

(mod7c_vu13p.tcl). BD validated with no critical warnings.

- 2026-10-13 Build #1/#2 (run_build.sh): IP OOC runs crashed with SIGBUS at Vivado start (parallel runs racing on

~/.Xilinx) → XILINX_LOCAL_USER_DATA=no added to run_build.sh.

- 2026-10-14 Build #2 impl result: (a) DRC UCIO-1 — refclk pins of X1Y5/X1Y6/X1Y7 missing from physical.xdc

(lost when the other session restored its 20:40 backup) → re-added AT11/AM11/AH11 (+ clock groups); stray ] on the

pma_rst_clk_p false-path line fixed. (b) Timing WNS −0.116 ns (104 endpoints) on 400 MHz inter-SLR paths

link_x1y8 RX FIFO → TX FIFOs of 224–227 → added axis_slr_x1y8_to_tx (mod9_vu13p.tcl).

- 2026-10-14 Build #3 (01:17–01:40, design with broadcaster ← M_AXIS_x1y8_1): synth/impl/bitstream complete,

vu13p.runs/impl_1/aurora_gty_vu13p_rst_wrapper.bit. Timing met: WNS +0.026 ns, WHS +0.010 ns, 0 failing endpoints.

GT placement checked (reports/gt_placement.txt): hier_x1y4..x1y11 on GTYE4_CHANNEL_X1Y16..X1Y47 / COMMON X1Y4..X1Y11,

13P↔13P on X0Y44-47 / X0Y52-55; all refclk pins equal the golden table above. DRC: only warnings (PDCN-1569 ×3,

RTSTAT-10 ×1). Methodology: TIMING-4/TIMING-27 critical warnings on clk_wiz_0/inst/clk_in1 (clk_wiz OOC clock

redefinition under sys_clk, inherited from the original project), LUTAR-1 ×95 warnings.

- 2026-10-14 09:5x User edited the top BD in the Vivado GUI (journal ~/Desktop/vivado.jou): broadcaster input

axis_slr_x1y8_to_tx/S_AXIS moved from M_AXIS_x1y8_1 (core1, X1Y34-35) to M_AXIS_x1y8 (core0, X1Y32-33).

BD validates clean. ⚠ Not consistent with z1, which still sends ADC ch00 on X0Y1 core1 (S_AXIS_X0Y1, lanes

X0Y6-7) — awaiting user decision (move z1 to core0, or revert here). Build #3 bitstream predates this edit.

- 2026-10-14 End-to-end check of the 1R8T data path (BD trace + RTL simulation scripts/sim_chain/run.sh, xsim,

real RTL: axis_packetizer → header_insertion_v2 → 2 Aurora hops w/ back-pressure gaps → header_extraction_v2 →

data_delay → FIFO 1024 → data_aligner). Verified: all Aurora cores Framing, 25.625 G, 2-lane, lanes as tabled;

tuser = {24'b0, timer[39:0]} on z1; 160 b → 128 b (+64 b header once per 1024-beat packet) → 160 b with tuser

rebuilt per beat; RFDC z1 ADC00 and z2 DACs both 3.84 GSPS ×3 dec/int = 1.28 GSPS, 5 IQ/clk @ 256 MHz.

Sim results: delay 512 and 200 → every sample on the DAC exactly delay+1 cycles after its z1 timestamp, no gaps,

no drops; 40-bit timer wrap passes cleanly. Limits found:

(1) delay − link latency must stay < ~1000 (z2 hier_rx FIFO depth 1024; delay 1400 → overflow, samples lost —

data_delay ignores tready); (2) losing 1 packet on any link → 64-bit word misalignment, garbage until reset

(packet = 64 + 1024×160 b = 1280.5 beats, the half beat is carried into the next packet); PACKET_LENGTH 1022 makes

packets beat-aligned and recovers; (3) a timestamp jump > delay+1024 (≥2 lost packets) freezes data_delay's

timestamp guard → z2 output stops until reset; (4) z1/z2 timers only agree if both boards' FPGA_clk share a reference

and both arstn are released before the same SYSREF pulse.

- 2026-10-14 Per user: broadcaster source back to X1Y8 core1 (mod10_vu13p.tcl), matching z1 X0Y1 core1. Validated.

- 2026-10-14 Per user ("naming is chaos … wire, hier, pin, port"): full naming pass (see *Naming convention*):

both containers (aurora_gty_vu13pto47dr: 377 renames + 522 wires; aurora_gty_vu13ptovu13p: 42 + 69 wires —

its S_AXIS_x0y13 actually fed core1) and the top BD (port reconnect, GT ports, 36 wires). Container port i_pma_rst

→ ini_pma_rst (same as 47DR). Old links e.g. M_AXIS1=TX core0, src_arst1=core0, every hier_x1yN/gt_x1y8_diff_ref

were misleading. Checks: every container port name equals its traced quad/core (80/80); net/endpoint counts identical

before/after; validate clean; all XDC ports present in the regenerated wrapper. Backup: backup/pre_naming_20261014.tgz.

- 2026-10-14 Per user: FIFO depths of every link read from Vivado (scripts/check/fifo_report.tcl): z1 TX

cdc/out 16/16, VU13P rx/tx 512/512, z2 in/cdc 512/512, z2 hold 1024. FIFO-budget simulation with header/framing

overhead and Aurora pause model (scripts/sim_chain/run_fifo.sh): link load ≈ 82.6 %, no loss in normal operation

or with a 1 µs VU13P TX stall (peak 321/512); a 3 µs stall overflows the VU13P TX FIFO. System design document

docs/DESIGN.md (rev 0.9) and deck docs/pj22045_1r8t_design.pptx written in the format of

~/Downloads/serdes_selecio_axis/docs.

- 2026-10-14 Per user: docs/DESIGN.md rev 0.10 — §6 *Time base and timers (whole system)* added; DESIGN.docx / deck regenerated.

- 2026-10-14 12:41 Per user: build #4 started (synth → impl → bitstream, run_build.sh vu13p 8, log scripts/build_vu13p.log) on the renamed rev 0.10 designs; results pending.

- 2026-10-14 Per user: debug ILAs (mod12_add_ila.tcl): ila_x1y4…ila_x1y11 in the container (TX C0/C1, RX C0/C1,

2048 deep, replaces the X1Y4-only system_ila_0) and ila_bcast at the top (broadcaster input + M00…M07).

- 2026-10-14 14:10 Build #5: bitstream + .ltx, WNS +0.025 ns, WHS +0.010 ns, 40 GTs and all refclk pins =

golden, DRC warnings only, methodology only TIMING-4/27 (inherited clk_wiz). LUT 3.3 %, FF 4.7 %, BRAM 18.4 %.

- 2026-10-14 Per user: sources not used by the top removed (D18): aurora.bd, 14 RTL files (slr_register*,

data_pg, header_*, axi_monitor, axis_adapter, data_mhk/shk/valider/viewer, work_persentage). Backup

backup/pre_cleanup_20261014.tgz. Top and BD validation unchanged; runs stay up to date.

- 2026-10-14 15:00 Release PJ22045_1R8T_release_rev0.11_20261014.tar.gz (45 MB, project root): bitstreams + .ltx, z1/z2 XSA, docs, reports, sources; all runs up to date, 1815 files SHA256-checked after extraction (scripts/release/make_release.sh).

- 2026-10-14 Per user: two detailed documents in the format of docs/DESIGN.docx: docs/SIGNAL_TRANSMISSION.md/.docx

(every module and IP configuration on the signal path, links, flow control, start-up, errors) and docs/TIMING_SYNC.md/.docx

(time base, SYSREF, stamping, header carry/rebuild, data_delay/data_aligner design, latency, alignment, failure modes,

bring-up). IP values read from a read-only dump of all BDs; scripts/docs/make_docx.py <NAME> now builds any of the three.

Findings (not fixed): PL_SYSREF has no input-delay constraint (timer start edge untimed); RTL analysis of data_delay guard /

data_aligner epoch corrects DESIGN.md §6.9 rows (z1 reset, z2 reset, delay change).

- 2026-10-14 DEFECT found (not fixed, awaiting user): in all eight aurora_gty_vu13pto47dr/hier_x1yN, aurora_rstn_1/channel_up

is driven by core 0 lane_up (net aurora_x1yN_0_lane_up) instead of aurora_x1yN_1/channel_up. Inherited from EV1 hier_x1y4

and copied to all quads by the rev 0.5 "all links identical" pass (D9) — now also on X1Y8 C1 (ADC stream). Fix = rewire 8 nets + rebuild.

- 2026-10-14 17:22 Per user ("fix 1 and rebuild"): D19 applied (scripts/mod13_vu13p_rstn.tcl, backup

backup/pre_rstn_fix_20261014.tgz): all eight hier_x1yN/aurora_rstn_1/channel_up now on aurora_x1yN_1_channel_up;

BD validated; built container copy checked 8/8. Build #6: WNS +0.011 ns, WHS +0.010 ns, 0 failing endpoints,

GT/refclk placement identical to rev 0.11 (golden). Bitstream + .ltx in vu13p.runs/impl_1/. The rev 0.11 release

tarball still holds the old VU13P bitstream. PL_SYSREF constraint not added (D20, user).

- 2026-10-14 17:33 Release PJ22045_1R8T_release_rev0.12_20261014.tar.gz (52 MB, 1843 files SHA256-checked): VU13P build #6 (D19), z1/z2 build #5, three documents + figures.

- 2026-10-17（Rex，v1.0.0_Debug7）雙向：axis_slr_x1y4_to_z1 + axis_broadcaster_1 + ila_bcast_1（X1Y4 C1 → X1Y8~11）；dbg_hub 改用 sys_clk 100 MHz；WNS +0.006 / WHS +0.010 ns。見文件開頭「v1.0.0_Debug7 修改」。